NETWORKING

### LAN & WAN – Quick Notes

#### LAN (Local Area Network)

- Small area (home, school, office)

- Connects devices locally

- Shares files, printers, and internet

- Example: Home Wi-Fi

#### WAN (Wide Area Network)

- Large area (cities, countries, worldwide)

- Connects multiple LANs

- Enables long-distance communication

- Example: The Internet

### Key Difference

- LAN = Small/local network

- WAN = Large network connecting LANs

### Switches, Routers & Firewalls – Quick Notes

#### Switch

- Connects devices within the same LAN

- Manages data flow between devices

#### Router

- Connects different networks

- Directs traffic between networks

- Example: Connects home network to the internet

#### Firewall

- Protects the network from unauthorised access

- Monitors and controls network traffic

### Key Points

- Switch = Connects devices

- Router = Connects networks

- Firewall = Protects the network

### IP & MAC Addresses – Quick Notes

#### IP Address

- Unique address for each device on a network

- Allows devices to communicate

- IPv4: 32-bit (e.g. 192.168.0.5)

- IPv6: 128-bit, more available addresses

#### MAC Address

- Unique hardware address for a network device

- Also called Media Access Control address

- 48-bit hexadecimal address

- Identifies devices on a local network (LAN)

### Key Difference

- IP Address = Identifies device on a network

- MAC Address = Identifies the physical network device

### Ports & Protocols – Quick Notes

#### Ports

- Logical numbered doors for network communication

- Direct data to the correct application

- Port 80 = HTTP

- Port 443 = HTTPS

#### Protocols

- Rules for sending and receiving data

- Ensure devices communicate correctly

- Examples: HTTP, FTP, SMTP

#### TCP (Transmission Control Protocol)

- Connection-oriented

- Reliable and accurate

- Uses a 3-way handshake

- Slower due to error checking

- Used for: Web browsing, email, file transfers

#### UDP (User Datagram Protocol)

- Connectionless

- Faster but less reliable

- No handshake or error checking

- Used for: Video streaming, online gaming, DNS, VPNs

### TCP vs UDP

- TCP = Reliable, slower

- UDP = Fast, less reliable

### OSI Model – Quick Notes

#### Why use the OSI Model?

- Standard framework for network communication

- Simplifies network management

- Allows independent upgrades to each layer

### 7 OSI Layers (Top → Bottom)

7. Application

- Provides network services to applications

- Examples: HTTP, FTP, SMTP

6. Presentation

- Translates and encrypts data

- Examples: SSL, TLS

5. Session

- Establishes, manages, and ends sessions

4. Transport

- End-to-end data delivery

- Protocols: TCP, UDP

3. Network

- Routes packets between networks

- Uses: IP addresses, Routers

2. Data Link

- Transfers data between devices

- Uses: MAC addresses, Switches

1. Physical

- Physical transmission of data

- Examples: Cables, Fibre, Wi-Fi

### Easy Memory Trick

All People Seem To Need Data Processing

- Application

- Presentation

- Session

- Transport

- Network

- Data Link

- Physical

### TCP/IP Model – Quick Notes

#### TCP/IP Model

- Backbone of the Internet

- Simpler version of the OSI model

- Has 4 layers

### 4 TCP/IP Layers

1. Application

- Network services and protocols

- Examples: HTTP, TLS, DNS

2. Transport

- End-to-end communication

- Protocols: TCP, UDP

3. Internet

- Routing and logical addressing

- Protocol: IP

4. Network Access

- Physical network connection

- Examples: Ethernet, Wi-Fi, LAN

### Key Points

- TCP/IP = 4 layers

- OSI = 7 layers

- TCP/IP is the model used on the Internet.

### Ports & Protocols – Short Notes

#### Ports

- Logical doors for network communication.

- Direct data to the correct application.

- Port 80 = HTTP, Port 443 = HTTPS.

#### Protocols

- Rules that control how data is sent and received.

- Examples: HTTP, FTP, SMTP, TCP, UDP.

#### TCP (Transmission Control Protocol)

- Connection-oriented (uses a 3-way handshake).

- Reliable: checks errors, resends lost data, delivers data in order.

- Slightly slower due to error checking.

- Uses: Web browsing, email, file transfers.

#### UDP (User Datagram Protocol)

- Connectionless (no handshake).

- Faster but less reliable (no guarantee of delivery).

- No error checking or retransmission.

- Uses: Video streaming, online gaming, DNS, some VPNs.

### TCP vs UDP

| TCP | UDP |
| --- | --- |
| Reliable | Fast |
| Connection-oriented | Connectionless |
| Error checking | No error checking |
| Web, email, files | Streaming, gaming, DNS |

Key Remember:

- Port = Door

- Protocol = Rules

- TCP = Reliable

- UDP = Fast

-   Ports are numbered logical doors that direct network traffic to the correct application (e.g., Port 80 = HTTP, Port 443 = HTTPS).

-   Protocols are rules that define how data is sent and received over a network.

-   The two main transport protocols are TCP (reliable but slower) and UDP (fast but less reliable).

-   TCP is used when accuracy is important, while UDP is used for real-time applications where speed matters.

### DNS (Domain Name System) – Short Notes

#### What is DNS?

- DNS (Domain Name System) translates domain names into IP addresses.

- Makes websites easier to access by using names instead of numbers.

#### How DNS Works

- Humans type a domain name (e.g., www.google.com).

- DNS finds the website's IP address.

- Devices use the IP address to communicate.

#### Why DNS is Important

- Makes the internet easier to use.

- Acts like a contacts list:

  - Name = Domain name

  - Phone number = IP address

Key Remember:

- DNS = Translates domain names → IP addresses.

- Computers use IP addresses; humans use domain names.

### DNS Components – Short Notes

#### Name Servers

- Store and manage DNS settings.

- Respond to DNS queries.

- Types:

  - Authoritative Name Server: Holds the actual DNS records and gives the final answer.

  - Recursive Name Server: Queries other servers to find the answer and caches results for faster future lookups.

#### Finding Name Servers

- Use the dig command:

  - dig NS google.com

  - dig +short NS google.com

- Displays the domain's name servers.

#### Zone Files

- Stored on name servers.

- Contain DNS records for a domain.

- Help name servers resolve domain names.

- Organise DNS information in a structured format.

Key Remember:

- Authoritative = Final answer.

- Recursive = Finds the answer.

- Zone file = Stores DNS records.

### DNS Resource Records – Short Notes

#### Resource Records (RR)

- Stored in zone files.

- Contain information about domains and DNS resources.

- Main components:

  - Name – Domain name.

  - TTL (Time to Live) – How long a record is cached.

  - Class – Namespace (usually IN).

  - Type – Record type (A, AAAA, MX, etc.).

  - Data – Information for the record (e.g., IP address).

#### Common Record Types

- A Record – Maps a domain to an IPv4 address.

- AAAA Record – Maps a domain to an IPv6 address.

- CNAME – Creates an alias for another domain.

- MX Record – Specifies the mail server for a domain (uses priority values).

- TXT Record – Stores text, commonly used for domain verification and security.

Key Remember:

- A = IPv4

- AAAA = IPv6

- CNAME = Alias

- MX = Email server

- TXT = Verification/Text data

### How DNS Works – Short Notes

#### DNS Resolution

- Converts a domain name into an IP address.

- Allows browsers to find the correct web server.

#### DNS Hierarchy

- Root Server – Points to the correct TLD server.

- TLD Server (.com, .org, .net) – Points to the authoritative name server.

- Authoritative Name Server – Stores the domain's DNS records and returns the IP address.

- Domain/Zone – Contains the zone file with DNS records.

#### DNS Query Process

- Browser checks local cache and hosts file.

- Sends request to a DNS resolver.

- Resolver checks its cache.

- If not found, queries:

  - Root Server

  - TLD Server

  - Authoritative Name Server

- Resolver returns the IP address to the browser.

- Browser connects to the website.

#### Registrar vs DNS Hosting Provider

- Registrar: Sells and registers domain names.

- DNS Hosting Provider: Hosts the DNS zone and DNS records.

- They can be the same or different companies.

Key Remember:

- Order: Local Cache → Resolver → Root → TLD → Authoritative → IP Address.

- Registrar = Buys the domain.

- DNS Host = Stores DNS records.

### DNS Debugging Tools – Short Notes

#### NSLOOKUP

- Basic tool for querying DNS records.

- Command: nslookup google.com

- Shows:

  - DNS server used.

  - IP address of the domain.

  - Whether the answer is authoritative or non-authoritative (cached).

#### DIG (Domain Information Groper)

- Advanced DNS query tool with more detailed output.

- Command: dig google.com

- Shows:

  - Question section (query made).

  - Answer section (DNS records/IP addresses).

  - Query time and other details.

- Useful commands:

  - dig +short google.com → Short output.

  - dig +short NS google.com → Displays name servers.

Key Remember:

- NSLOOKUP = Simple DNS lookup.

- DIG = Detailed DNS troubleshooting.

- DNS uses Port 53.